State Machines and the LCD Display

| Module | ET4047 |
| --- | --- |
| Date | 28/10/2026 |
| Lab No. | Lab 6 |
| Author | Luke Griffin |

Figure 1: Adafruit RGB LCD shield wiring: 5V, GND, SDA → A4 and SCL → A5

# 1  Task 1: A traffic light on the LCD

Objective: run a three-state traffic light (RED → GREEN → YELLOW) and change the state machine.

## Wiring

I2C LCD connections (used in every task)

| From |  | To | Purpose |
| --- | --- | --- | --- |
| LCD GND | → | Nano GND | Ground |
| LCD VCC | → | Nano 5V | Power |
| LCD SDA | → | Nano A4 | I2C data |
| LCD SCL | → | Nano A5 | I2C clock |

Figure 2: I2C backpack LCD wiring. For the RGB LCD shield, see Figure 1.

Figure 3: Task 1 state diagram

Code Upload Task1_Traffic_Light. Use the folder for your display: I2C_Backpack_LCD (separate screen with four wires) or Adafruit_RGB_LCD_Shield (the shield with five buttons).

Expected result: the LCD shows State: RED for 5 s, State: GREEN for 5 s, State: YELLOW for 2 s, and repeats.

## Task 1 questions

### 1a  Run it

Upload the code and check the states change in the right order. (record) Take a photo of the LCD.

### 1b  The transition table

Fill in the table from the code.

| Current state | Time in state | Next state | LCD shows |
| --- | --- | --- | --- |
| RED |  |  |  |
| GREEN |  |  |  |
| YELLOW |  |  |  |

How long does one full cycle take?

Your answer:

### 1c  Change the timing

Make YELLOW last 3 seconds instead of 2. Which line did you change? Why is it better to keep the times in named constants such as YELLOW_TIME than to type delay(2000) inside the case?

Your answer:

### 1d  Add a state

Add a fourth state, RED_AMBER, which shows State: RED+AMBER for 2 s, so the cycle becomes RED → RED_AMBER → GREEN → YELLOW → RED. Which three places in the code did you change? (record) Draw your new state diagram.

Your answer:

# 2  Task 2: Traffic light with real LEDs

Objective: drive a red, a yellow and a green LED from the state machine, with exactly one LED on at a time.

## Wiring

LED connections (keep the LCD wiring from Task 1)

| From |  | To | Purpose |
| --- | --- | --- | --- |
| Nano D5 | → | 220 Ω → red LED long leg | Red light |
| Nano D6 | → | 220 Ω → yellow LED long leg | Yellow light |
| Nano D7 | → | 220 Ω → green LED long leg | Green light |
| Each LED short leg | → | GND rail → Nano GND | Common ground |

Figure 4: Task 2 wiring

Figure 5: Task 2 LED outputs over two cycles

Code Upload Task2_Traffic_Light_LEDs. Use the folder for your display: I2C_Backpack_LCD (separate screen with four wires) or Adafruit_RGB_LCD_Shield (the shield with five buttons).

Expected result: one LED at a time, in step with the LCD (State: RED / STOP, and so on).

## Task 2 questions

### 2a  Run it

(record) Paste a few lines from the Serial Monitor. They should look like this:

Serial Monitor

Traffic light ready State 0: RED State 1: GREEN State 2: YELLOW

### 2b  Forget to switch an LED off

In each case, replace the setLights(…) line with a single digitalWrite(…, HIGH) for that state’s LED. Run it for two cycles. What do the LEDs show, and why? Compare with Figure 5. Then put setLights() back.

Your answer:

### 2c  The numbers behind the names

What would the LCD show if you changed the enum to { RED, YELLOW, GREEN } but left the two text arrays as they are? Why?

Your answer:

### 2d  The resistor

A red LED drops about 2 V and the pin gives 5 V. Roughly how much current flows through the 220 Ω resistor?

Your answer:

# 3  Task 3: A pedestrian crossing

Objective: the cars’ light stays GREEN until the button is pressed and released, then goes YELLOW → WALK (countdown) → CLEAR → GREEN. The sketch uses millis() instead of delay(), and the button is debounced and reacts on the rising edge (when it is released).

## Wiring

Add the button (keep the LEDs and LCD from Task 2)

| From |  | To | Purpose |
| --- | --- | --- | --- |
| Nano D2 | → | Push button, one leg | Pedestrian button |
| Push button, other leg | → | GND rail | Pressing pulls D2 to 0 V |

Figure 6: Task 3 wiring

Figure 7: Task 3 state diagram

Code Upload Task3_Pedestrian_Crossing. Use the folder for your display: I2C_Backpack_LCD (separate screen with four wires) or Adafruit_RGB_LCD_Shield (the shield with five buttons).

Expected result: the light stays GREEN until you press and release the button; the LCD shows WAIT..., then a WALK 6s … 1s countdown.

## Task 3 questions

### 3a  Run it

Press the button. (record) Paste the Serial Monitor output for one crossing. It should look like this:

Serial Monitor

Crossing ready -> GREEN Button released -> YELLOW -> WALK -> CLEAR -> GREEN

### 3b  Press it at different times

Press and release the button at each of the times below. Use the state diagram to explain each result.

| When you press and release | What happens | Why |
| --- | --- | --- |
| Just after the light turns GREEN |  |  |
| When GREEN has been on for more than 5 s |  |  |
| During WALK |  |  |
| During CLEAR |  |  |

### 3c  delay() would miss it

If Task 2 checked the button at the top of loop(), what is the longest a pedestrian might have to hold it before the board noticed? Why does Task 3 notice a tap of a few milliseconds?

Your answer:

### 3d  Rising edge and debouncing

1. Press the button and keep holding it. When does WAIT... appear: when you press, or when you let go? Which edge of the D2 signal (rising or falling) is the sketch reacting to?

2. In buttonReleased(), a new reading only counts once it has stayed the same for DEBOUNCE_TIME (50 ms). Why is this needed for a mechanical button?

3. Set DEBOUNCE_TIME to 0 and press the button several times. Does anything change? Would it matter if the button started a different action on every press, such as counting pedestrians?

Your answer: